Server Driver for NetWare 4.01

Introduction:

-------------

This document describes the procedure to install the NetWare v3.12

server driver for Genius LAN GE2500III SE PCI ethernet adapter.

Location of Driver:     \NETWARE\NW40X\PCISRV.LAN

Installation Procedure:

-----------------------

Before you start with the installation  process,  make sure that

the Novell NetWare v3.12 server is properly installed.

Similarly, your adapter should also be properly installed in your

server.

1. These files can be obtained from Genius LAN GE2500III SE driver disk subdirectory

A:\NETWARE\NW40X

MSM.NLM

ETHERTSM.NLM

2. If you can log into the server as a supervisor , copy PCISRV.LAN

relative files from the Genius LAN GE2500III SE Ethernet Card

Driver Diskette subdirectory A:\NETWARE\NW40X into the NetWare 386

subdirectory SYSTEM of your server. (If some files exists, rename

the existing files in the SYSTEM subdirectory).

3. When the NetWare server prompt appears (indicated by a colon),load

your server driver. Simply type:

: UNLOAD ETHERTSM <Enter>

: UNLOAD MSM      <Enter>

: LOAD ETHERTSM <Enter>

: LOAD PCISRV <Enter>

or, you can directly load file from Genius LAN GE2500III

PCI Ethernet Card :

: UNLOAD ETHERTSM <Enter>

: UNLOAD MSM  <Enter>

: LOAD A:\NETWARE\NW40X\ETHERTSM <Enter>

: LOAD A:\NETWARE\NW40X\PCISRV <Enter>

4. Bind IPX to the adapter driver. At the NetWare server prompt

,type:

:BIND IPX TO PCISRV <Enter>

5. After pressing <Enter> the computer prompts you for the Network

Number. For details on how to assign this number,please consult

your NetWare 386  Installation Manual.

6. Add the load and bind statements you require to the server's

AUTOEXEC.NCF file so that the LAN driver will load automatically

each time the server starts up.

NOTES:

------

1. Installing Multiple LAN Adapters:

The keyword  "SLOT"  is  provided for multiple LAN adapters in a

single server by the driver PCISRV.LAN.  So, add  "SLOT" in LOAD

commands. For example:

LOAD PCISRV FRAME=Ethernet_802.2 NAME=LAN_A SLOT=1

BIND IPX TO LAN_A NET=11

LOAD PCISRV FRAME=Ethernet_802.2 NAME=LAN_B SLOT=2

BIND IPX TO LAN_B NET=22

2. Installing One LAN adapter with Multiple Frame Types:

When  binding multiple frame types to one adapter, enter a LOAD

and BIND statement for each frame type. Each LOAD statement will

use a different network number is required on the bind statement.

You need  to supply  a name on each load line in order to avoid

being  prompted  for which  board to bind IPX to. If you do not

have the name option in the AUTOEXEC.NCF , it will not  execute

completely without user intervention.  Example:

LOAD PCISRV FRAME=ETHERNET_802.3 NAME=IEE8023

BIND IPX TO IEE8023 NET=11111

LOAD PCISRV FRAME=ETHERNET_802.2 NAME=IEE8022

BIND IPX TO LAN8022 NET=22222

If problems occur during the binding process, your  screen will prompt

you with error  messages.  Please refer to your NetWare 386 manual for

details on these messages.