Workstation for DOS ODI Client

Introduction:

-------------

This document describes the procedure to setup the NetWare v3.X , v4.X

workstation driver for Genius LAN GE2500III SE PCI ethernet adapter.

Location of Driver:     \NETWARE\DOSODI\PCIODI.COM

Sample Configuration Files:

---------------------------

STARTNET.BAT:

LSL

PCIODI

IPXODI

NETX or VLM (VLM's for NetWare 4.x )

NET.CFG

LINK DRIVER PCIODI

FRAME ETHERNET_802.3

EtherID 0123456789AB   ------>  This keyword  is Ethernet

address of a adapter, and

only  required  when more

than one GE2500III SE

adapters exist on one

system

Setup Procedures for NetWare 3.11/3.12 Client :

-----------------------------------------------

Before you start with the installation process, make sure that

the  adapter is properly installed and configured. Make sure

that your NetWare workstation is properly installed.

1. Copy  PCIODI.COM from the Genius LAN GE2500III SE Driver

Diskette subdirectory A:\NETWARE\DOSODI to your harddisk.

2. Create a batch file, or add to your STARTNET.BAT file the commands

listed above under STARTNET.BAT

3. Edit the NET.CFG file that you copied over. Edit the file per your

requirements (see examples above).  If there are two frame types

listed, whichever frame type is listed first, that is the one that

the  driver will load.  See above for examples.  The NET.CFG file

should be in the same directory with the LSL.COM file.

Setup Procedures for NetWare 4.X Client with vlm:

-----------------------------------------------

The  installation utility  INSTALL.EXE  is  located on NetWare's

"Workstation for DOS" disk. Run INSTALL.EXE from the Workstation

for DOS Disk.

The NetWare Client Install program screen will appear.  You must

complete each of the questions.  These options are specific to your

needs.

1. Enter the destination directory for the NetWare Client.

(The default directory is C:\NETWARE\DOSODI).

2. Gives you the option to automatically update the CONFIG.SYS

and the AUTOEXEC.BAT files or modify them at a later time

manually.

3. Do you wish to install support for MS Windows ? (Y/N)

(MS Windows Subdirectory: default C:\WINDOWS)

4. Asks you to select a network driver and provide  configuration

options.  It uses  the specifications you select to create the

file NET.CFG. Insert the Genius LAN GE2500III SE drivers disk

when prompted for the driver disk and specify the driver path

A:\NETWARE\DOSODI

5. Press <Enter> to install

NOTE:

-----

1. Be sure to add LASTDRIVE=Z to your CONFIG.SYS file for the

network mappings to work properly.

2. Installing Multiple LAN Adapters:

The keyword, EtherID, is provided for multiple LAN adapters in a

single system by  PCIODI.COM. If EtherID unknown, you can ignore

it on the  NET.CFG  file and directly run  PCIODI  under the DOS

prompt,then your screen will  display  every adapter EtherID and

select which one you need.